CHAPTER 5

FLUID DYNAMICS

5.1 Introduction

In discussing about hydrostatics we were concerned with forces (pressure forces) which are acting on an object for a liquid at rest and when we deal with kinematics of fluid flow phenomena related with space time variation (velocity and acceleration) with out considering the effect of force. But in dealing with dynamics of fluid flow, all forces that affect the phenomenon are considered.

The dynamics of fluid motion deals with kinetics, which relates the kinematics with the forces responsible for causing the motion. This relation ship of fluid motion is established by the use of laws of nature.

- The principle of conservation of mass (the continuity relation ship)

- Newton’s laws of motion

- The 1st and second laws of thermodynamics

5.2 Equations of motion

The dynamic behavior of fluid motion is governed by a set of equations, which is known as equations of motion. These equations are obtained by using the Newton’s second law, which may be written as Fx = m.ax

Where, Fx is the net force acting in the x-direction upon a fluid element of mass m producing an acceleration ax in the x-direction.

The forces which may be present in fluid flow problems are gravity forces Fg, pressure force Fp, force due to viscosity Fv, force due to turbulence Ft, Surface tension Fs, and force due to compressibility of fluid Fc.

Gravity forces (Fg.) is due to the weight of the fluid. Its component in flow direction results in acceleration.

Pressure force (Fp): It is equal to the product of pressure intensity and cross sectional area of the flowing fluid. Acts normal to the surface under consideration and produces acceleration in the given direction.

Viscous forces (Fv): - Exists in real fluids. It is the shearing resistance generated when there is relative motion between two layers of fluids. It acts opposite to the direction of motion, and retards flow.

Surface tension (Fs): This force is important when the depth of flow is extremely small.

Force due to compressibility (Fc): for incompressible fluids, this becomes significant in problems of unsteady flow like water hammer. In most of flow problems, Fs and Fc are neglected.

Force due to turbulence (Ft): the continuous momentum transfer between layers in highly turbulent flow results in normal and shear stresses known as Reynolds's stress.

If the changes for the change in forces are small the forces can be taken negligible.

m.ax = (Fg)x+(Fp)x+(Fv)x+(Ft)x

The presence of such a complex system of forces in real fluid flow problems makes the analysis very complicated. Therefore, mathematical analysis of problems is generally possible only if certain simplifying assumptions are made.

5.2.1 Energy and Head

A liquid in motion may possess three forms of energy.

- Potential energy /elevation /positional energy/ because of its elevation above datum level. If a weight w of liquid is at a height of z above datum

Potential energy   = Wz

Potential energy per unit weight   = z (meters) = Potential head

2.  Pressure energy:  When a fluid flows in a continuous stream under pressure it can do work. If the area of cross – section of the stream of fluid is a, then force due to pressure p on cross- section is Pa.  If a weight w of liquid passes the cross section.

- Volume passing in cross section = W/ρg

Distance moved by liquid   = Wρga

W = F* S = Pa∗Wρga = WPρg

Pressure energy per unit weight   = Pρg  = pressure head.

3.  Kinetic energy

If a mass of fluid (m) moves at some velocity (v),

Kinetic energy = ½ mv2 = ½ W/g v2

Kinetic energy pr unit weight = v22g = kinetic head

Total head = potential head + pressure head + velocity head = Z+Pγ+v22g

Bernoulli’s theorem states that the total energy of each particle of a body of fluid is the same provided that no energy enters or leaves the system at any point. The division of this energy between potential, pressure and kinetic energy may vary, but the total remains constant.

In symbols            Z+Pγ + v22g = constant

5.3 Bernoulli’s Equation

Consider a cylindrical element of stream tube having cross-sectional area dA length ds unit weight  as shown in motion along a streamline.

dsdA

PdA



ds

Z

Fig: 5.1 Pressure and gravity forces on a cylindrical element along a streamline

The normal forces on the side faces are in equilibrium and as the fluid is assumed non-viscous, there is no shear stress. The velocity varies along the streamline and there is acceleration. It is necessary to take into account force due to acceleration when considering the longitudinal balance of force.

But in the ease of steady flow the velocity doesn’t vary at a point so that local acceleration will be zero ∂v∂t=0 but for velocity variation with position convective acceleration will be different from zeroV∂v∂s≠0. The forces tending to accelerate the fluid mass are pressure force on the two ends of the element,

[Fs = mas] Summation of force in the arbitrary‘s’ direction.

PdA−p+dpdsds dA =−dp dA

Weight in the direction of motion,

−ρgdsdAcos θ=−ρgdsdAdzds=−ρgdAdz

dsdA

PdA



ds

Z

Applying Newton’s 2nd Law of motion F=ma

−dPdA−ρg dA dz=ρds dAVdvds    as as = ∂v∂t + ∂v∂s V                                    dpρ+gdz+Vdv =0  One-dimension Euler’s equation

It can be applied for both compressible and incompressible flow.

dpγ + dz+Vgdv = 0

For the case of an incompressible fluid  may be treated as constant, the integration gives

dpγ + dz+VdVg = Constant

Pγ + Z+V22g = Constant [Bernoulli’s Equation]

Under special conditions the assumption underlying Bernoulli’s equations can be waived.

- When streamlines originate from a reservoir,

- For unsteady flow with gradually changing conditions (E.g. Emptying a reservoir) the equation may be applied without appreciable error,

- It may be used for real fluids, by modifying the result experimentally.

The Bernoulli equation is the basis for the solution of a wide range of hydraulics problems. For two points along a streamline, the Bernoulli equation may be expressed in the form of:

y1+P1γ + v122g = y2+P2γ+v222g

5.3.1 Bernoulli’s Equation for real fluid

The Bernoulli’s equation expressed by pγ+Z+v22g is determined for an incompressible ideal fluid without taking in to account the effects of some other forces as viscous, etc.

In case of real fluid these forces should be introduced so that the equation needs some modification.

A real fluid does possess viscosity and consequently it offers resistance to flow. In order to overcome this viscous resistance and other resistances due to surface roughness and turbulence, some part of the total energy of the flow is lost. [Energy is neither created nor destroyed but may be changed to heat energy increasing temp of the fluid]

The increase in temperature of the fluid causes an increase in the internal energy. The increase in internal energy and the heat transfer from the fluid represent a loss of useful energy. The total loss per unit mass of fluid is (u2-u1-q).

Energy loss per unit weight in overcoming resistance hl=u2−u1−qg(head loss)

The total energy of flow decreases in the flow direction, and consequently the energy line has a down ward slope.

Horizontal

2

1

V22/2g

h1

P2/

Z2

V12/2g

Z1

P1/

Datum

The modified Bernoulli’s equation for up stream section (1) and downstream section (2)

V122g+P1γ+Z1=V222g+p2γ+Z2+hl

hl = head loss between the two sections.

Z1

HL = head loss

Z2

HGL

EGL

Datum

Fig.5.2 Energy & hydraulic grade lines

5.3.2 Energy correction factor

The analysis of flow problems is usually based on the one-dimensional approach. The entire flow is considered to be taking through stream tube with average velocity V at center of each cross-section. The velocity distribution at any x-section in real fluid flow is non uniform, on account of the boundary resistance and consequently the kinetic energy per unit weight given by V2/2g doesn’t represent the kinetic energy across the section.

Velocity distribution

Real liquid flow

Ideal liquid flow

Vavg

dA

In order to compensate for the discrepancy a coefficient known as energy correction factor denoted by  is used. The multiplication of  with V2/2g yields the kinetic energy actually passing a section.

For the figure given,

The kinetic energy per unit time passing through on elemental area dA is ½ (dAu)u2

u - Velocity at that point

Total kinetic energy passing the section

A12ρu3dA And the actual kinetic energy passed on average velocity V passing the section is equal to α12ρV3A

From the two equation

α=1Auv3dA

Kinetic energy correction factor  is a measure of viscous resistance generated in a given flow, the effect of which is reflected uniform nature of velocity distribution. For a given pipe, it can be shown that its magnitude is a function of the type of flow and its turbulent characteristics.

Laminar flow is purely a viscous flow; the value of  is maximum and equals 2.0. But in case of fully developed turbulent flow in pipes,  is independent of Reynolds number and may be considered to have almost constant value (1.01 to 1.15) depending on surface roughness and Reynolds number. Lower value is appreciable for velocity rough surface and high Reynolds number.

α1V122g+p1γ+Z1=α2V222g+p2γ+Z2+hℓ

For an identical velocity distribution at two sections, 1 = 2, and if accuracy is not required in non-uniform velocity distribution  = 1.

##### 5.3.3 Practical Applications of Bernoulli’s Equation

Bernoulli’s equation is applicable in all problems of incompressible fluid flow where energy considerations are involved. In addition, it is practically applied for flow measurement using the following measuring devices.

1. Venturimeter (tube) (G.B Venturi (1746 –1822) Italian Eng)

It is a device used for measuring rate of flow in a pipeline and it consists of three components:

- A converging entrance cone of angle of about 200

- A cylindrical portion of short length called the “throat”

- A diverging section known as diffuser, of cone angle 50 to 70 to ensure a minimum loss of energy, but where this is unimportant the angle may be as large as 140.

The entrance tube and exit tube diameter are the same as that of the pipe line in to which it is inserted and the length of throat is equal to the throat diameter.

Fig.5.4 Horizontal Venturimeter

Assuming that the fluid is ideal (So that energy is not dissipated in overcoming frictional resistance) and that the velocities V1 and V2 at the inlet and throat respectively, are uniformly distributed over the cross section (so that the energy correction factor  is 1). Applying Bernoulli’s equation between points (1) and (2) on a central stream line and assuming no frictional resistance,

V122g+P1γ+Z1=V222g+P2γ+Z2

For an incompressible fluid, continuity of flow at section 1 and 2 is,

Q = A1V1 = A2V2

V2=A1A2V1 ……………………….(a)

If A1 > A2, V1 < V2   i.e. KE at section 2 (throat) > KE at section 1(the entrance)

P at throat < P at entrance

For a horizontal Venturimeter,

V22−V122g = P1−P2ρ g ......(b)Substituting equ (a) in to eqn (b)V122g A1A22−1 = P1−P2ρ g V1 = 1A1A22−1 ∗ 2g ∗ P1−P2ρ g

V1=A2A21−A222g P1−P2ρg

Theoretical Discharge

Qt = A1 V1t =A1A2A21−A222gP1−P2ρ g

The theoretical discharge can be converted to actual discharge (Qa) by multiplying

Qa = Cd∗Qt=CdA1 V1t =CdA1A2A21−A222gP1−P2ρ g

The differential pressure is evaluated from manometry, and for figure.5.4

P1γs+ys−xs−sm(y−x)=P2γsP1−P2γ=x(sms−1)

For vertical or inclined Venturi meter, the actual discharge can be computed similarly.

- Pitot tube (Total head tube)   / (Henri Pitot)

Pitot tube is a device used for measuring velocity of flow at any point in a pipe or a channel. In its elementary form a Pitot tube consists of an L-shaped tube with open ends. Its may be aligned in open channel or pipe flow measurement as indicated below.

###### Stagnation pressure

For a figure below it has been seen that the central streamline terminates at B the entrance to the Pitot tube. This is on account of the inability of the streamline to take a sudden turn. The fluid flowing along the central streamline, therefore, stops moving as it reaches the point B. Hence the velocity of flow at this point is zero. This point is known as stagnation point.

Applying the Bernoulli’s equation at points A and B, we obtain

PB = PO = PA+VA2/2. The pressure at the stagnation point is known as stagnation pressure composed of static pressure PA and dynamic pressure VA2/2.

If the measurement is made on an open channel flow the surface will be exposed to the air and there is no static head from the surface, and if measurement is made on pipe flow there will be static head at A.

Fig.5.5 Simple Pitot tube in pipe flow and open channel flow

Applying Bernoulli’s equation to point A in the undisturbed flow region and the stagnation point B we have

VO22g + POγ + ZO = VA22g + PAγ + ZA⇒ POγ = VA22g + PAγ VA22g = PO−PAγVA2 = PO−PAγ2g ⇒ VA= 2g (PO−PAγ) = 2gh since PO−PAγ=h

A perfect Pitot tube should obey this equation exactly, but all actual instruments must be calibrated and a correction factor applied to make allowance for the small effects of nose shape and other characteristics.

Practically it is difficult to read h from a free surface. To overcome this difficulty, the static tube and the Pitot tube may combine in one instrument (differential U-tube).

For finding the velocity at any point in a pipe by Pitot tube, the following arrangements are adopted.

- Pitot tube along with a vertical piezometer tube as shown in Fig.5.6

- Pitot tube connected with piezometer tube as shown in Fig.5.7

- Pitot static tube, which consists of two circular concentric tubes one inside the other with some annular space in between as shown in Fig.5.8.

The outlets of these two tubes are connected to the differential manometer where the difference of pressure head is measured by knowing the difference of the levels of the manometer liquid hm.

Pitot tube

Piezometer Tube

h

Fig.5.6

Fig.5.7

Fig.5.9

The Pitot tube measures the velocity of only a filament of liquid, and hence it can be used for exploring the velocity distribution across the pipe cross-section. If, however, it is desired to measure the total flow of fluid through the pipe, the velocity must be measured at various distances from the walls and the results integrated. The total flow rate can be calculated from a single reading only if the velocity distribution across the cross-section is already known.

The static tube measures the static pressure, since there is no velocity component perpendicular to its opening and the impact tube measures both the static and impact pressure (due to kinetic energy). Impact tube head = pressure head + velocity head.

- Orifices

An orifice is an opening (usually circular) in the wall of a tank or in a plate normal to the axis of a pipe, the plate being either at the end of the pipe or in some intermediate location and used for measuring rate of flow out of a reservoir (tank) or through a pipe.

a. Orifice flow in pipes, Orifice meter or orifice plate

The Venturi meter described earlier is a reliable flow-measuring device. Furthermore, it causes little pressure loss. For these reasons it is widely used, particularly for large-volume liquid and gas flows. However, this meter is relatively complex to construct and hence expensive. Especially for small pipelines, its cost seems prohibitive, so simpler devices such as orifice meters are used.

The orifice meter consists of a flat orifice plate with a circular hole drilled in it. There is a pressure tap upstream from the orifice plate and another just downstream.

Fig.5.9 Orifice meter

Fig.5.10 Orifice plate in a pipe

Applying the Bernoulli equation between at 1 (upstream of plate) and 2 (at the orifice)

P1γ + V122g = P2γ + v222g Since the orfice is horizontal Z1=Z2V12−V222g = P2−P1γUsing continuity equation A1v1 = A2 V2 ⇒ V2 = A1A2 V1  v12−A1A2 2v122g = P2−P1γ v12 1−(A1A2)22g = p2−p1γ ⇒ V1 = p2−p1γ∗2g1−(A1A2)2

The theoretical discharge Q will therefore be,

Q = A1V1 =A1 p2−p1p∗ 21−(A1A2)2

The actual discharge will be less than the theoretical since the effective flow area near P2 tapping will be less than A2, the fluid forming a neck or vena contracta. In addition, there will be some loss of energy between point 1 and 2.

The actual discharge can be determined by determining coefficient of discharge.

b. Flow through a reservoir opening (orifice flow)

For a reservoir at water level h above orifice opening shown in the following figure. The reservoir is assumed to be very large as compared to size of the opening, so that the velocities of all points in the reservoir are negligibly small.

3

Fig. 5.11 Small orifice in the side of a large reservoir

Applying Bernoulli’s equation and neglecting losses between A and B (taking the datum level at the center of the orifice),

0 + 0 + h   = V22g + 0 + 0

V = 2gh (Theoretical velocity). Where h is the depth of center of orifice below the free surface. Evidently, the velocity of efflux is equal to the velocity of free fall from the surface of reservoir. This is known as Torricelli’s theorem. In real fluid flows, there is always some loss of energy due to viscous effects and accordingly the actual velocity will be less than the theoretical velocity.

#### Hydraulic Coefficients for flow through orifices

1) Coefficient of contraction, Cc = Area of jet at vena contractaArea of orifice

2) Coefficient of velocity,      Cv = Actual velocity at vena contractaTheoretical velocity=V2gh

3) Coefficient of discharge,    Cd = Actual dischargeTheoretical discharge

Theoretical discharge, Qt = Ao 2gh

Where Ao = Area of orifice(Area of jet at C)

Actual velocity, Va = Cv 2gh

As shown in the figure, the paths of the particles of the liquid converge on the orifice so that the area of the issuing jet is less than the area of the orifice. In the plane of the orifice, the particles have a component of velocity towards the center so that at C the pressure is greater than atmospheric pressure. It is only at B a little outside the orifice that the paths of the particles become parallel. The section at B is the least cross-section and hence of maximum contraction, is called the vena contracta.

Area of vena contracta,     AB = CcAo	  (The actual area of the jet is the area of the vena contracta not the area of the orifice.)

Therefore, Actual discharge Qa = Actual area of jet * Actual velocity

Qa=CcAo∗Cv2gh

It is customary to combine the two coefficients into a discharge coefficient Cd.

(Cd = Cv Cc)

Qa = Cd A0 2gh

Determination of hydraulic coefficients of orifice

Determination of Cd

By measuring area A0, the head h and the discharge Qa (by gravimetric or volumetric means), Cd is obtained using the above equation. Determination of either Cv or Cc then permits determination of the other by the equation Cd = Cv Cc.

Determination of Cc and Cv

- Trajectory method

The equation for the trajectory may be obtained by applying Newton’s equation of uniformly accelerated motion to a particle of the liquid passing from the nozzle to point P, whose coordinates are (x, y) in time t. Then x = Vxot and z = Vzot - 1/2gt2. Evaluating t from the first equation and substituting in the second gives

z=Vz0Vx0x−g2Vx02x2

If the jet is initially horizontal, as in the flow from a vertical orifice, Vx0 = V0 and Vz0 = 0, the above equation is reduced in to;

V0 = xg2z

Then Cv = V0Vt = V02gH = x2zg2gh = x4zh

- Pitot tube method

Pitot tube can be set at the vena contracta so that actual velocity Va is determined.

- Calipers method

The diameter of the jet at the vena contracta can be approximately measured using an outside caliper. However, this method is not precise and is less satisfactory than other methods.

#### Types of Orifice

#### The following figure shows common types of orifice with their coefficient of discharge.

#### Fig.5.12 Types of orifice

The orifices are classified based on of their size, shape, nature of discharge and shape of the upstream edge.

- Depending upon their size: - small orifice and large orifice; If the head of liquid from the center of the orifice is more than five times the depth of the orifice, the orifice is called small orifice. If it less than five times it is known as large orifice.

- Depending upon shape: - as circular, triangular, rectangular and trapezoidal

- Depending upon shape of edge: - as sharp edged and round or bell mouthed orifice. (Fig.5.14)

- Depending up on the nature of discharge: - as free discharging & drowned or submerged orifice; the submerged orifices are further classified as fully submerged and partially submerged orifice

3. Unsteady orifice flow from reservoirs

The volume discharged from the orifice in time t is Qt, which must just equal the reduction in volume in the reservoir in the same time increment t. AR(-y), in which AR is the surface area of the reservoir at level y above the orifice.

y

y

Fig 5.13 Unsteady flow from reservoir

Equating the two expressions:

Qδt = AR(−δy)Solving for δt and integrating yields δt = AR(−δy)Q dt = − AR δyQ but Q = Cd Ao 2gy ∴ t = −ARCd Ao2g y−12δy t = 2ARCd A0 2g(y112 − y212) This is the time for the liquid to fall from y1 to y2.

- Weirs

Open channel flow may be measured by a weir or obstruction in the channel that causes the liquid to back up behind it and flow over it or through it. There may be sharp crested or broad crested based on their length along the channel section.

a) Rectangular weir

The following figure shows rectangular notch of crest length (L) and working under a head H.

Fig. 5.14 Rectangular notch

By Torricelli’s theorem, the velocity of a particle discharged at any level h is 2gh and will therefore vary from top to bottom of notch. Considering a horizontal strip at depth h and of thickness h

Discharge through strip dQ = 2gh * Lh

Total discharge Qt=L2 g0Hh12dh =23L2gH32

This is the theoretical discharge through a rectangular notch.

The value of Q given by the above equation is too high because no account has been taken of energy lost and also because, as shown below there will be a substantial reduction in the width and depth of the notch cross section because of the curved path lines of the liquid.

Fig 5.15 Effect of vena contracta in rectangular weir

Therefore, Actual discharge Qa, Qa=23Cd L2gH32

From experiment, Cd = 0.62

Qa = 1.831LH3/2

b) Triangular weir (V-notch)

Fig. 5.16 Triangular weir

Since the velocity of flow through the notch varies from top to bottom, consider a strip of thickness h at a depth h below the surface. If the velocity of approach is small:

Head producing flow = h

Velocity through strip = V = 2gh

If width of strip = b, Area of strip = bh

Discharge through strip = Q = Vbh

The width b depends on h and is given by b = 2(H - h) tan

δQ=(2g)h12x2(H−h)tanθ ∗δh =22gtanθ(Hh12−h32)δh

Integrating between the limits h = 0 and h = H

Q=22gtanθ0H(Hh12−h32)dh =22gtanθ23Hh32−25h520H =8152gtanθH52

This is the theoretical discharge.

Actual discharge = Cd*Q = Cd8152gtanθH52

From experiment Cd = 0.58, Qa = 1.37 tan H5/2,

For 900 V-notch, Qa = 1.37 H5/2

  - Impulse-momentum theorem

It is often important to determine the force produced on a solid body by fluid flowing steadily over it. For example, the force on a pipe bend caused by the fluid flowing through it; the force exerted by jet of fluid striking against a solid surface; thrust on a propeller. All these forces are hydrodynamic forces and they are associated with a change in the momentum of the fluid.

The magnitude of such a force is determined by Newton’s second law of motion, by modifying the law to suit particularly to the steady flow of a fluid called the steady flow momentum equation. Only the forces acting at the boundaries of this space concern us, and use of momentum equation doesn’t require the knowledge of the flow pattern in detail. Moreover, the fluid may be compressible or incompressible and the flow with or without friction.

Consider a stream tube shown below with the following assumptions

- The c/s of stream tube is sufficiently small so that the velocity may be considered uniformly distributed

- The flow is steady i.e. the stream tube remains stationary with respect to the fixed coordinate axis.

Fig.  Stream tube

Newton’s second law

F = m * a

F = m dvdt

F * dt = m * dv

- Momentum principle expresses that the rate of change of momentum is equal to the net force acting on the fluid mass.

Momentum of fluid entering section 1 –1 in a time t in the x –direction

=  * dQ * t * V1(x)

Momentum leaving section 2- 2 in time t

=  * dQ * t * V2 (x)

From momentum principle

dFx   = P dQ Δt [v2(x) − v1(x)]Δt

dFx = dQ [V2 (x) – V1 (x)]

dFx = net force exerted on the fluid in the x –direction.

The total force in the x –direction is given by

Fx = dFx = ρdQ v2(x)−ρdQ v1(x) = ρ (v2dA2) v2 (x) − ρ(v1dA1) v1(x)

Assuming the fluid is incompressible

Fx = V2 A2 (V2 (x) - PV1A1 V1 (x)

Fx =  Q [V2(x) – V1(x)]

Similar equations for y and z directions may be written

Fy = Q [V2(y) – V1(y)]

Fz = Q [V2(z) – V1(z)]

## Applications of momentum equations

1) The force caused by a jet striking a surface

### A. Impact on a flat surface

i)  Stationary plate:

A = area of jet

v1 = velocity of jet

 = mass density

Fig. A jet hitting a vertical plate

Fx = Q(v2 (x) – v1(x))

= Q (0 – v1)  momentum normal to the plate is destroyed.

Fx = -AV2   (force in the jet)

Force exerted on plate = AV2 (Equal and opposite to the force exerted on the jet = Fx)

ii) Moving plate

Initial velocity of jet v1x = V1

Final velocity of jet = velocity of = v2x = U = velocity of plate

Fig. A jet hitting a moving vertical plate

The velocity with which jet strikes the plate = V – U

Mass of fluid striking plate/sec   = A (V – U)

Force on plate   = A (V – U) (V – U)

= A (V – U) 2

iii) Stationary inclined plate

Fig. A jet hitting an inclined plate

Initial velocity of jet v1(x) = Vsin

Final velocity of jet v2(x) = 0

Mass striking stationary plate = AV

Normal force on the plate = AV(Vsin) = AV2 sin

Since the plate surface is smooth, there can be no force exerted by the plate on the fluid jet in the tangential direction.

Ft = Q1V - Q2 V – PQVcos  = 0

Q1 – Q2 – Qcos = 0 (*)

Continuity equation    Q = Q1 + Q2    Q2 = Q – Q1

Substitute in (*)                                        Q1 = Q - Q2

Q1 – Q + Q1 – Qcos  = 0

2Q1 - Q (1+ cos) = 0                       Q – Q2 – Q2 – Qcos = 0

Q1 = Q/2 (1 + cos)                          Q (1 - cos) – 2 Q2 = 0

Q2   = Q/2 (1- cos)

For a vertical plate,   = 90o

Q1 = Q/2 = Q2

iv) Moving inclined plate

Fig. A jet hitting a moving inclined plate

Initial velocity of jet V1(x) = Vsin

Final velocity of jet V2(x) = Usin

Mass striking the moving plate per second   = Q = A (V- U)

Normal force on the plate   = A (V - U) (V sin - U sin)

= A (V – U) 2 sin

Work done by this force = Fx * U = F n sin θ ∗u

V) Series of flat plates

The force exerted by the impact of jet can be fruitfully utilized if the flat plates are mounted on the periphery of a wheel as shown below. The force exerted by the jet causes the rotation of the wheel. The flat plates thus occupy the bottom most position according to their turn.

Fig. Flat plates mounted a wheel

The number and location of the plates is so arranged that no portion of jet goes waste without doing work on the plate.

Initial velocity of jet = V

Final velocity = velocity of plate = U

Fluid mass striking the plate per second   = Q = AV

Force exerted on the plate by jet = AV(V – U)

Work done per second    = Force * distance moved per second

= AV (V –U) *U

KE of jet per second = ½ (AV)V2

Efficiency of jet   = Work done on plateKE of jet = ρAV(v−u)u12 ρAV V2 = 2(v−u)uv2

#### B. Force on curved vane

Consider a symmetrical curved vane having smooth surface. The jet strikes at the center and after impact it is deflected equally along the vane surface.

Fig. A jet hitting a curved vane

Min    = AV * V = AV2

Mout = - ρAv2 Vcosθ − ρ Av2 vcos θ

= - AV2cos

Force exerted by curved vane on the jet

Fx = Mout – Min

= - AV2cos - AV2 = - AV2 (1+cos)

Force on curve vane by jet = AV2(1+cos)

For  = 900   – Flat plate at right angle to jet   F = AV2

For  = 00       - semi–circular plate   F = 2AV2

### Curved vane moving in translation

Mass of fluid striking the vane = A(V- U)

Min = A(V - U) 2

Mout = - A(V – U) 2cos

Force exerted by jet on the vane = Fx = A (V – U) 2 (1+ cos)

Work done by jet = FxU

KE of jet /sec = ½ (AV)V2

Efficiency, η = workdone by jetKE of jet

η = 2 (1+cosθ) V−U2UV3

Curved vane mounted on a wheel

Let the wheel rotate with a tangential velocity U and a jet moving with velocity v strikes the wheel.

Fluid mass striking the wheel per second = PAV

Min = AV(v –u) (Before impact)

Mout = AV(v –u)cos (After impact)

Force exerted by jet on the van = Min – Mout

F = AV (v – u) (1+ cos)

W = F * U

KE = ½ AV2     η = 2u (v−u) ( 1+cosθ)v2

2.  Force exerted on a reducing pipe bends

i. Bend in a horizontal plane.

Y

X

θ

Fig. Force on a horizontal reducing bend

Let Fx and Fy be the components of the force exerted on the fluid by the pipe bend.

Then momentum equation in the x–direction can be written as:

P1A1 - P2A2cos - Fx = Q(V2cos - V1)

Fx = Q (V1 -V2cos) - P1A1 + P2A2cos

Momentum equation in the y–direction can be written as:

Fy - P2 A2sin = pQ (V2sin - V1)         (V1 = 0)

Fy = P2A2sin + pQ(V2sin)

ii. Bend in a vertical plane.

A reducing bend with deviation in the vertical plane is shown in Fig. below. Due to the hydrostatic and dynamic pressures a force is exerted by the fluid on the bend which has to be resisted by a thrust block or other suitable means. This force could be evaluated by plotting the stream lines and thus determining the pressure distribution. However, by a simple application of the momentum equation, and quite independently of any energy losses associated with turbulent eddying (real fluid), we obtain.

Z

Y

θ

Fig. Force on a vertical reducing bend

For the x direction:

P1 A1 - P2A2cos  - Fx = pQ(v2cos - v1)

And for the z direction

Fz - W – P2A2 sin   = pQv2 sin

Where W is the weight of fluid between the reference sections.

From these equations, Fx and Fz may be determined and hence the resultant force, R:

R = Rz2+Rz2

It is to be noted that the momentum equation gives no information concerning the location of the resultant, which necessitates an analysis involving forces and moments.

3. Force in nozzle

A nozzle, attached to a pipeline, and discharging to the atmosphere provides a good example of a rapid change in velocity. The fluid exerts a force on the nozzle and in accordance with Newton's third law there is a similar force, of opposite sign, exerted by the nozzle on the fluid. This is the force which the tension bolts must be designated to withstand. The force could be evaluated by an analysis of the pressures acting on the surface of the nozzle but this procedure would, to say the least, be extremely tedious. By contrast, a simple application of the momentum equation between upstream and downstream reference sections will yield a direct solution. In Fig below, the component forces are the hydrostatic forces P1A1 and P2A2 and the force Fx exerted by the nozzle on the fluid. The rate of change of momentum is ρQ(V2 - V1) so that the momentum equation may be expressed as:

Fx = P1A1 - ρQ(V2 - V1)